| John Arlinger | 2024-05-29 |
| --- | --- |

Standardization of operational monitoring in forest machines

Definitions AND implementations of time concepts and repaIr causes within StanForD 2010

###### Contents

Introduction	1

Background	1

General	2

Time monitoring	3

Definition and structure of time concepts	3

Machine indicators	8

Starting and ending registration of different type of time intervals	11

Flow chart	12

Filtering of time registration	13

Short down times	14

Registering past and future time	14

Causes for repair	15

Causes for disturbances	18

Example – harvester application	18

Other issues	20

Registration of working-hours	20

F.A.Q.	21

References	21

Appendix 1. StanForD variables	1

Appendix 2. Enumeration lists for repair causes	1

Appendix 3. Enumeration lists for disturbances and maintence.	1

Appendix 4. Swedish interface.	1

Appendix 5. Illustrations of machine parts.	1

Appendix 6. Minimum filter time and machine indications	1

Appendix 7. Swedish instruction for time registration in operational monitoring of forest machines	1

Basic principles	1

Introduction	1

Registration of down times	2

Examples	4

# Introduction

This document is a revised version describing operational monitoring reporting using version 4.0 of mom-schemas or higher. This document deviates significantly from the previous standard versions (1.0 -3.6) as coding of down times is more flexible and the way data is to be stored is more streamlined.

The main updates are:

- Both CombinedWorkTimes and FrequencyShortDownTimes have been excluded. These were previously used by at least one manufacturer.

- Codes for DownTimes can be controlled using a separate schema called OperationalMonitoringCodeList (file type mcl).

- The old StanForD2010CodeList schema has been removed from the standard.

- OperatorLoginTime structure replaced by the OperatorLogInEvent structure.

- New element IndividualMachineWorkTimeKey added to IndividualMachineWorkTime.

- TrackingCoordinates, ShortDownTimes, OperatorWorkTimes extended with element IndividualMachineWorkTimeKey.

# Background

A standardised system for operational monitoring of forest machines is a vital tool when analysing the performance and efficiency of different forest machines.

The monitoring system described in this document is quite complex. It is therefore strongly recommended that any person trying to implement operational monitoring contact the StanForD secretary in order to make sure that nothing has been misunderstood and that new functions has not been added.

This document describes the definitions of time concepts and causes for repairs and disturbances within the framework of StanForD. It also describes how to implement these concepts in forest machines in order to make it possible to interpret the results in a fair and comparable way independently of machine system.

StanForD is an abbreviation for Standard for Forest machine Data and communication. The standard, which was implemented in 1988, is a global de-facto standard for forest machines. Further information concerning the standard can be found at Skogforsks homepage (https://www.skogforsk.se/stanford/). The information in this document is to a large extent also included in the StanForD 2010 xml schemas.

The first StanForD variables related to operational monitoring of forest machines were implemented with the first version of the standard in 1988. Additional variables were added in April 1999, based on a proposal from Partek Forest, in order to make the monitoring of forest operations more flexible. These variables were 315, 316, 317, 318 and 319.

In 2001 the Swedish Forest Engineering Cooperation Group (TSG) proposed a set of new and stricter definitions of time concepts that should be included in new a practical system for monitoring of forest operations. These suggested definitions were to a certain extent adjusted according to the report Time concepts for machine work. Skogsarbeten, (Anon 1969)

During 2002 and 2003 a working group consisting of John Arlinger, (StanForD secretary, Skogforsk), Lennart Hult (Sveaskog), Bo Morenius (StoraEnso), Sören Bergek (SCA) och Bengt Larsson-Snygg (SMF) wrote a proposal for a new monitoring system. The group discussed this proposal with Tomas Johansson (SCA), Jörgen Eriksson (ESE-Technique) as well as with the board of SMF (organisation for Swedish forest entrepreneurs). A Swedish seminar was also arranged in May 2003 for all major manufacturers and service providers for forest machines where the proposal was discussed. This document is based on this proposal which was presented/implemented in October 2003.

The Nordic agreement on forest work study nomenclature (Anon. 1978) was studied when developing the present standard. However, since this agreement did not focus on forest machines in practical operation it was deemed to be of limited use. The agreement is also predominantly focused on detailed scientific studies rather than practically applicable methods for day to day monitoring of forest operations.

During 2013 the document was re-written in order to be in accordance with the new StanForD 2010 standard which is based on xml-format. The first StanForD 2010 version 1.0 was completed in June 2011.

A major update of the xml schemas for operational monitoring was carried out with the introduction of version 4.0. This document has hence been significantly updated in 2023 and 2024.

All relevant xml-schemas can be downloaded at http://www.skogforsk.se/stanford2010.

The term monitoring is used in the document as a synonym to operational monitoring of forest machines.

The document was originally written for wheeled harvesters and forwarders where it is assumed that the controller has is integrated with both the attachment and the base machine. This means that tracked machines are not covered to any significant extent. The same is true for machines where the controller and base machine are manufactured and “sold” separately and there is limited communication between the controller and the base machine.

# General

The objective of operational monitoring of forest machines is to enable analyses of the influence of technical and organisational factors on the efficiency of forest machines (harvesters and forwarders).

- The system for operational monitoring makes it possible to compare and follow up:

Different machine systems/models

Reasons for repairs, e.g. which components that cause the machine to frequently stand still due to repair

Different organisations

Different types of harvesting sites

Variation over time

- The standard presently covers a wide variety of machines as illustrated below:

- UHarvestersU are, in this document, defined as machines which main purpose is to fell and process (including bucking to value) stems into logs.

- UForwardersU are defined as machines which main purpose is to transport the processed logs from the harvesting site to the road side. It does not have the capability to fell and process stems.

- FellerBunchers are defined as machines used only for felling single or multiple stems without any bprocessing into logs.

- Skidders are defined as machines used for pulling whole stems from the stump to a landing by dragging the stems behind the machine.

- Yarders are defined as machines used for pulling stems using a system of cables to pull or fly logs from the stump to a landing.

It UmustU be possible to store the monitoring report (mom-file according to StanForD) for a certain time interval, meaning that it should be possible to send, for example, a daily or weekly report from the machine for analysis at the office. The time interval for reporting must be flexible. It UshouldU also be possible to generate a monitoring report per harvesting object. It UshouldU, in other words, be possible to generate reports that are Uboth object and time oriented.

It must be possible to generate a mom-file for a whole harvesting site irrespectively of how long time it took to harvest the site, but it must also be possible to send a report for a shift or a month.

Several controllers have implemented the possibility of automatically generating a mom report per hour. It has been noted that this type of reporting is complex to do in a correct way as the present activity may now be known, for example if a down time started at three minutes before a file is to be generated. This down time may at a later stage e.g. be defined as a short down time or not. It is therefore strongly recommended that it is possible to generate production data (fpr or hpr) once an hour but mom-files only once per shift or work day. Typically it is probably of little relevance to report mom-files for a shorter time period than a shift as fairly random events may affect the results when analyzing very short time intervalls.

Since there are legal restrictions concerning automatic registration of the working time and performance of individual employees in certain countries, it UmustU be possible to Ude-activateU the monitoring system. It is also UrequiredU that it is possible for the machine owner/operator to control whether or not the mom-file is sent. The machine owner should, in other words, never be forced by the control system to send the hpr- and mom-files together; the machine owner/operator must have total control over the reporting of mom-files.

# Time monitoring

## Definition and structure of time concepts

Definitions and structures of time concepts are described in this section. The StanForD elements where the data is to be stored are also presented. A complete listing of all relevant StanForD elements and attributes can be found in appendix 1.

All basic time concepts below (table 1) are stored in individual StanForD elements with the exception of Calendar time, Utilised time, Lost time and Main work time. Time intervals are normally to be registered individually in IndivudalMachineWorkTime in the mom-file (figure 1a). However, it is also possible to register UsummedU times per concept and operator in CombinedMachineWorkTime. This means that it is permitted to register an unlimited number of time intervals in a mom-file.

Figure 1a. Main structures for registering machine times in mom-file.

Lost time is defined as Calendar time minus Utilised time. This is the time during which the machine is normally not working, for example during nights and weekends. With the exception of meal breaks during a work day, no time intervals must be registered as Lost times. This means that Lost time is calculated by subtracting Utilised time from Calendar time. Observe that Lost time can not be divided up per operator with the present definitions, neither can consequently Calender time. The following example describes how Lost time is handled:

A machine has not been working for three days due to repair. Normally it is working for 16 hours per day. In this case 48 hours (3*16) should be registered as Repair time and the Calendar time will be 72 hours (3*24). This means that the Lost time will be 24 hours (72-48).

Some additional aggregated/grouped concepts that are calculated using the concept in table 1 are described in table 2. These are not stored in the mom-file. A number of calculated key figures used for evaluating and comparing different systems are presented in table 3. These are to a large extent defined according to Time concepts for machine work. Skogsarbeten, (Anon 1969). Notice that two of the key figures should not be calculated per operator.

There is no StanForD elements for recording Calendar time, however it can be estimated by calculating the difference between the start time and the end dates recorded in element ReportInterval (includes ReportStartTime, ReportEndTime and ReportFilterCategory).

Observe that USwedish namesU for central time concepts and key figure UmustU be according to appendix 4 in Swedish applications for recording, administrating and analysing monitoring data. This list of codes for different down times is significantly shorter than previously as version 4 is much more flexible with the inclusion of the new mcl-file.

Table 1. All basic time concepts, also described in StanForD appendix, time intervals can be recorded as a sum per operator or each time can be recorded individually in the mom-file

| Name | Description | StanForD elements | Swedish translation |
| --- | --- | --- | --- |
| Calendar time (Ca) | All available time, for example one work week = 7 * 24 h = 168 h, can be calculated through ReportInterval (incl ReportFilterCategory) |  | Kalendertid |
| Utilised time (To) | Total time required to carry out a certain task, that is Effective work time plus Down time Normally paid work time intended for work on machine. |  | Utnyttjad tid |
| Lost time (L) | Calendar time which is not utilised time (Ca-To, for example meal break or night time when no work is to be carried out) |  | Outnyttjad tid |
| Effective work time (E) | Time required to perform a specified work element which directly or in directly changes the work object in regard to its form, position or state. E(t) where t is according to MonitoringFilterTimeDown. |  | Grundtid |
| Main work time (M) | Time when machine carries out work for which it is intended. Divided into Processing and Terrain travel |  | Egentlig arbetstid |
| Processing (Mp) | Harvester: processing (felling and bucking), Forwarder: loading and unloading Harwarder: processing and loading/unloading | MachineWork-TimeCategory | Upp-arbetning |
| Terrain travel (Mt) | Harvester: Machine time in terrain, primarily for transport between depot and work place (stand / site / object) Forwarder: Machine time in terrain for transporting logs, excl. loading and unloading Harwarder: Machine time in terrain, primarily for transport between depot and work place or for transporting logs | MachineWork-TimeCategory | Terräng-körning |
| Other work (O) | Other work that is done by machine, for example salvaging / towing other machine, laying out branches on strip roads etc. Following otherWorkCategories exist. - Road travel: Moving machine by it self between different harvesting objects / sites. Preparation before and after moving is included - Preparing strip road: preparing a strip road, e.g. by placing branches and tops on to the strip road - Towing of other machine: the present machine is towing or salvaging another machine - Roadside loading of truck: Forest machine loading trucks at road side. This means loading whole stems or logs on to trucks. - Unspecified: | MachineWork-TimeCategory, otherWorkCategory | Övrig arbetstid Flyttning Risning stickväg Bärgning Lastning lastbil Övrigt |
| Repair time (Drep) | Repair, adjustments of machine, transportation to repair shop | MachineDownTime, Repair | Reparation |
| Maintenance (Dm) | Machine oversight/maintenance according to manufacturer’s instruction. Control and calibration of measuring system included | MachineDownTime, Maintenance | Skötsel |
| Disturbance (Dd) | Disturbances stopping the machine when it otherwise should have been active, e.g. due to machine getting stuck, extreme weather, ordered restrictions of production, visits, pre-harvest planning | MachineDownTime, Disturbance | Störning |
| Other downtime (Do) | Moving machine by trailer between harvesting objects / sites. Preparation before and after moving is included. Only time when operator is present (paid work time), e.g. no recording of time at night outside paid working time. | MachineDownTime, OtherMachineDown-TimeCategory | Övriga avbrott |
| Waiting for repair time (Drw) | Waiting for repairman or spare part, no machine activities (code from mcl) | MachineDownTime, OtherMachineDown-TimeCategory | Väntan reparation |

Table 2. Aggregated time concepts

| Name | Description |
| --- | --- |
| Effective work time (E) | Mp + Mt + O |
| Total repair time | Drep + Drw |
| Down time | Drep + Dm + Dd + Do (Drw is part of Do) |
| Utilised time | Effective time + Down time |

Table 3. Methods for calculating key figures used for evaluating and comparing different systems (* should not be calculated per operator as calendar time can not be divided operator)

| Name | Description |
| --- | --- |
| Percent utilisation* | (Utilised time / Calendar time) * 100 |
| Percent technical utilisation | (Effective work time / Utilised time) * 100 |
| Percent effective work utilisation* | (Effective work time / Calendar time) * 100 |
| Percent down time | (Down time / Utilised time) * 100 |
| Percent repair time | (Total Repair time / Utilised time) * 100 |
| Percent machine availability | (Effective work time / (Effective work time + Repair time + Maintenance)) * 100 |
| Percent operational availability | (Effective work time / (Effective work time + Total repair time + Maintenance)) * 100 |

The relationships between the different time concepts are described in figure 1 and 2. The differences in how the Main work time is handled for a harvester vs. for a forwarder is described in figure 3 and 4. In the case of a harwarder/combination machine processing and loading are recorded in the same variable (Mp). Observe that E(t) is always to be equal to the sum of Main work time, Other work Other work (unspecified), Road travel, Preparing strip road, Towing other machine and Roadside loading of truck.

The main difference between Terrain travel and Road travel is that Terrain travel is an activity taking place within one specific site while Road travel is defined as the transportation of a machine between two different sites. Terrain travel is defined as machine time in terrain, primarily for transport from depot to work place / or transportation of logs while Road travel is defined as moving machine by it self between different harvesting objects / sites including preparation before and after.

Figure 2a. Bar chart describing the structure of the time concepts in table 1, observe that both repair, maintenance and disturbance are further divided up into sub-categories.

Figure 2b. Flow chart describing the relationships between the time concepts in table 1. 
* Codes are controlled through the mcl-file which means that the coding may very well vary froom market to market depending on the needs of the users of mom-data.

Figure 3. Flow chart describing the division of Main work time for harvesters.

Figure 3. Flow chart describing the division of Main work time for forwarders.

E0-time is basically the total effective work time excluding all Down times (failures) while the Effective time recorded according to table 1 includes all Down times shorter than t (traditionally 15 minutes, E15), it is normally denoted with E(t). The relation between Effective work time, and E0-time is described in figure 5. UShort down timesU (<t) that are excluded from E0 but included E15 shall be logged in a separate element structure (IndividualShortDownTimes). The recording of short down times shall be done so that the number of down times per time class (per minute) can be reported which is described in more detail in section “Short down times”.

Figure 5. Flow chart describing the relation between Effective time, E(t), and E0 (Net effective time). The number of short interruptions is recorded in variable 320, the total short down time can be calculated by subtracting E0 from E(t).

It is, according to the standard possible to change the down-filter-time, t, which is normally 15 minutes. See also section Filtering of time registration for more detailed information.

## Machine indicators

A system for operational monitoring UmustU automatically distinguish between three time concepts using the following definitions:

- Processing / loading & unloading –

  - Harvester: harvesting head and linkage active.

  - Forwarder: grapple and loader active.

  - Bundler: grapple, loader and feeding in bundler unit active.

- Terrain travel – machine moving, harvesting head/grapple or loader/linkage in-active.

- Down time – machine not moving, harvesting head/grapple or loader/linkage in-active. Observe that the operator has to manually specify the type of down time, e.g. repair or maintenance (see section Causes for repair).

Observe that a certain hierarchic order exists concerning Processing, Terrain travel and Down time. The time cannot, for example, be registered as Terrain travel or Down time if, at the same time, harvesting head and loader/linkage are active. This means that Processing always overrides Terrain travel and Down time and that Terrain travel always overrides Down time.

The machine indicators used in monitoring are defined below (illustrated in appendix 5):

- Harvesting head – indication through the feed rollers, meaning that feed rollers are in motion (turning). If not possible to use feed rollers any relevant signals to the head may be used in combination with activity in the linkage.

- Grapple – indication through the grapple cylinder, meaning that the cylinder piston is moving.

- Loader/linkage – indication through the slew (on loader) or swing (on cab or linkage), meaning that the slew or swing is in motion (turning).

- Machine moving – indication through the function upstroke of propeller pump (transmission pump).

If harvester head does not have feed rollers other feeding/holding devices should be used. A multi tree head used for extracting energy assortments may for example use the grapple if feed rollers do not exist. In the case of a scarifier the loader/linkage and head are replaced by activity in the scarification equipment. Contact StanForD secretary if further questions arise.

Below are some examples of how the machine indicators are handled in order to automatically distinguish between Processing / Loading & unloading, Terrain travel and Down time:

Time is registered as Processing for a harvester if feed rollers and swing are in motion (turning).

Time is registered as Loading & unloading for a forwarder if grapple cylinder piston and slew are in motion.

Time is registered as Terrain travel for a harvester if  upstroke of propeller pump (transmission pump) occurs while either feed rollers or swing are not in motion.

Time is registered as Terrain travel for a forwarder if  upstroke of propeller pump occurs while either grapple cylinder piston or slew are not in motion.

Time is registered as Down time  for a harvester if no upstroke of propeller pump occurs while either feed rollers or swing are not in motion.

Time is registered as Down time for a forwarder if no upstroke of propeller pump occurs while either grapple cylinder or slew are not in motion.

The following true/false tables (4-9) illustrate how the machine indicators are to be automatically interpreted.

Table 4. Processing harvester.

| Feed rollers | Swing | Processing |
| --- | --- | --- |
| Off | Off | False |
| On | Off | False |
| Off | On | False |
| On | On | True |

Table 5. Terrain travel harvester.

| Processing | Upstroke of propeller pump | Terrain travel |
| --- | --- | --- |
| False | Off | False |
| True | On | False |
| False | On | True |

Table 6. Down time harvester.

| Processing | Terrain travel | Down time |
| --- | --- | --- |
| False | False | True |
| True | False | False |
| False | True | False |

Table 7. Loading & unloading forwarder.

| Grapple | Slew | Loading & unloading |
| --- | --- | --- |
| Off | Off | False |
| On | Off | False |
| Off | On | False |
| On | On | True |

Table 8. Terrain travel forwarder.

| Loading & unloading | Upstroke of propeller pump | Terrain travel |
| --- | --- | --- |
| False | Off | False |
| True | On | False |
| False | On | True |

Table 9. Down time forwarder.

| Loading & unloading | Upstroke of propeller pump | Down time |
| --- | --- | --- |
| False | False | True |
| True | False | False |
| False | True | False |

Observe that the machine is considered to be, for example, moving for the entire minimum-filter-time (default value is 15 seconds) if an upstroke of the propeller pump (transmission pump) is registered once during the minimum-filter-time. The minimum-filter-time, which can be viewed as the smallest time unit used in the monitoring system, is further described in section Filtering of time registration.

The Uautomatic registrationU according to above is UignoredU if the operator has chosen to Umanually registerU the time as Other work (unspecified), Road travel (moving between harvesting objects), Preparing strip road, Towing other machin or Roadside loading truck. The time will, for example, be registered as Other work independently of machine activity until another type of time has been selected by the operator.

It is also possible to implement a function whereby the operator manually turns off all indicators, thus forcing the machine to register time as a down time. This is especially of interest if the disturbance “Unnecessary terrain work” is to be used in the machine.

## Starting and ending registration of different type of time intervals

The following definitions for starting and ending registration of time are valid for UharvestersU:

- Processing (Mp) is normally registered automatically when both loader/linkage and harvesting head are active (see previous section).

- Terrain travel (Mt), is normally registered automatically when machine is moving at the same time as loader/linkage or harvesting head are inactive.

- The operator can, while registering Main work time (Processing or Terrain travel) or Down time, chose to indicate the time as Other (unspecified), Road travel, Preparing strip road, Towing other machine or Roadside loading of truck. The operator must manually indicate when he wants to return registration to Processing or Terrain travel or Down time. If the operator has indicated that, for example, he is doing Other work  the system will continue to register the time as such independently of machine activity.

- A Down time is assumed if the machine is not moving while loader/linkage or harvesting head is in-active. The operator must, when resuming Main work time (normally after 2 minutes), manually define the type of down time and in some cases the reason for the down time, if the down time has been longer than the down-fitler-time (normally 15 minutes).

- The operator should be reminded that he probably should register the time as Processing if stems are processed (harvesting head is active) and time is registered as Other work(unspecified), Road travel (moving between harvesting objects), Preparing strip road, Towing other machin or Roadside loading truck.

The following definitions for starting and ending registration of time are valid for UforwardersU:

- Loading & unloading (Mp) is normally registered automatically when both loader/linkage and grapple are active (see previous section).

- Terrain travel (Mt), is normally registered automatically when machine is moving at the same time as loader/linkage or grapple are inactive.

- The operator can, while registering Main work time (Loading & unloading or Terrain travel) or Down time, chose to indicate the time as Other (unspecified), Road travel (moving between harvesting objects), Preparing strip road, Towing other machin or Roadside loading truck .The operator must manually indicate when he wants to return registration to Loading & unloading or Terrain travel or Down time. If the operator has indicated that, for example, he is doing Other work  the system will continue to register the time as such independently of machine activity.

- A Down time is assumed if the machine is not moving while loader/linkage or harvesting head is in-active. The operator must, when resuming Loading & unloading or Terrain travel (normally after 2 minutes), manually define the type of down time and in two cases the reason for the down time, if the down time has been longer than the down-fitler-time (normally 15 minutes).

## Flow chart

Registration of time can be described as in figure 8.

Figure 8. Example of a flow chart describing time registration. Two-way arrows indicate that the operator can manually “go” in both direction, e.g. from registering time as Road travel to Other work or Loading truck. An “event” means that the system automatically senses an indication. Selection of down time means that the operator manually has to define type of down time.
1) Loading and unloading in the case of forwarders.
2) Linkage and harvesting head active/in-active in the case of harvesters, loader and grapple active/in-active in the case of forwarders.
3) Time will be registered as Other work (e.g. Road travel, Preparing strip road, Towing other machine or Roadside loading truck) irrespectively of machine activities, the operator has to manually change type of time in order to register time differently.

## Filtering of time registration

In order to avoid operators being forced to define the reason for Down times in the case of short down times some kind of filtering is needed. The following filtering should be used by the monitoring system:

- UDown-filter-time (t)U: A Down time must exceed this filter time in order to be registered as a Down time. The suggested default value is 15 minutes. It is registered in MonitoringFilterTimeDown.

- UMain-filter-timeU: A Main work time (Processing, Terrain travel, Other work (unspecified), Road travel, Preparing strip road, Towing other machin or Roadside loading truck), must exceed this filter time in order to be registered as a new Main work time. The suggested default value is 2 minutes. It is registered in MonitoringFilterTimeRun. 
This means that if a Main work time is shorter than 2 minutes this time will be register as the previously register type of time.

- UMinimum-filter-timeU: A time (Main work time or Down time) being below this time is not to be consider at all. This filter time can be viewed as the smallest time unit used in the monitoring system. The suggested default value is 15 seconds. It is to be registered in MonitoringFilterTimeMinimum.
This means that, for example, a Down time shorter than 15 seconds will be included in th E0-time while a Down time longer than 15 seconds will not be included in the E0-time.

Figur 8b. Schema structure for registering filtering settings.

It UmustU be possible to Uset and changeU the different Ufilter timesU in the machine and the used filtering times UmustU be registered in the mom-files.

Two different examples are described in figure 6 and 7 to illustrate how the filtering is supposed function.

Figure 6. Example of filtering (Total time: E0 = 12 min & 10 sec, Main work time (E15) = 15 min & 10 sec, Down time = 16 min)

Figure 7. Example of filtering (Total time: E15 = 23 min & 20 sec, Processing = 11 min & 20 sec, Terrain travel = 12 min).

## Short down times

Short down times are defined as:

- Failures shorter than ”Down-filter-time”, normally 15 min (MonitoringFilterDown)

- Failures longer then ”Minimum-filter-time”, normally 15 sec (MonitoringFilterTimeMinimum)

Short down times are to be registered as as individual times in IndividualShortDownTimes.

Figure 7b. Schema structure for registering individual or combined short down times.

Observe that no down times shorter than 15 seconds (as defined by MonitoringFilterTimeMinimum) are ever to be registered.

## Registering past and future time

When Main work time starts after Utilized time a manual dating backwards is needed, this means that the operator manually indicate that the working time, that is Utilized time, was started before the machine was started. The same applies when finishing the registration of time. If, for example, the work shift of an operator starts with Maintenance which does not include starting the machine.

It is therefore suggested that the operator, when logging into a machine computer, has to specify the unregistered time since the previous work shift as Lost time, Meal break or Down time and perhaps vice versa when logging out of the system.

# Reasons for repair down times

The operator UmustU define the Ureason for RepairU when defining an Down time as a Repair. The selection of the causes for repair is done in three steps: Section of machine (1), Type of part (2) and Part (3). For example Carrier – Mechanics - Brake system or Harvesting head – Hydraulics – Motor for chain-saw. The codes used when defining the reasons are included in an mcl-fil. The mcl-file is based on the OperationalMonitoringCodeList-schema. The default codes, which are identical with the previous standard version, are presented in appendix 2 (StanForD_Default_v1_English.mcl). Level 1 and 2 (Section of machine and Type of part) are to be compulsory in all monitoring systems while the code on level 3 (Part) are defined through the mcl-fileU. Figure 9 describes how the selection of causes for repair is structured.

Figure 9.Example describing how the selection of causes for repair is structured, element Air under element CarrierRepairReason will include value “Other” if the operator do not select a Part (only Carrier – Air) while the on value will be “Pipe”  if Part is selected (Carrier – Air – Hose).

The information is to be stored in the structure for down times as described below in figure 9b and 9c.

Figure 9b. Schema structure for registering all types of down times.

Figure 9c. Schema structure for registering repair down times.

Figure 9d. Example illustrating how IndividualMachineDownTime is to be implemented.

# Reason for disturbances

The operator UmustU define the Ureason for DisturbanceU when registering a Down time as a Disturbance. This is only done in one step unlike the case of Repair time.

The codes used when defining the reasons are included in an mcl-fil (default values presented in appendix 3) and registered in DisturbanceStandardCode. Figure 10 describes how the selection of causes for disturbance is structured.

Figure 10. Example describing how the selection of reason for disturbances is typically structured.

The information is to be stored as illustrated in previous section, figure 9d.

Observe that code “Unproductive terrain work” is only to be registered if indicators are turned off manually by the operator.

# Example – harvester application

The following text describes an example of events that are analysed and registered by the monitoring system in a harvester during one day. Main work time is registered with a Down-filter-time of 15 minutes (E15).

- The operator arrives at the harvesting site at 07.00 and starts with 1 hour of maintenance .

- The machine is started at 08.00, operator logs into the computer and the monitoring system is automatically started. The operator specifies that he started with 1 hour of Maintenance (Im) prior to starting the machine. This means that the Utilised time started at 07.00.

- The harvester is driven from the landing/parking to the location of harvesting site. The time is automatically registered as Terrain travel since the machine has been moving, with no activity in loader/linkage and harvesting harvesting head, for more than 2 minutes (Main-filter-time). The operator has at all times the possibility to manually chose to register time as Other work (unspecified), Road travel, Preparing strip road, Towing other machin or Roadside loading truck.

- The harvester arrives at the location of the harvesting site. Harvesting is started. The time is automatically registered as Processing when both loader/linkage and harvesting head, has been active for more than 2 minutes (Main-filter-time).

- Work is discontinued due to a saw motor breakdown and the harvester is therefore shut down awaiting spare parts.

- The operator is asked for the reason of down time when machine is restarted after repair (3 h, 30 min) since the standstill was longer than 15 minutes (Down-filter-time). The operator specifies that he Waited for repair for 2 hours and 30 minutes and that the following Repair took 1 hour. The operator then has to define the reason for Repair, the reason is, in this case, defined as Harvesting head – Hydraulics – Saw motor (code 3204).

- Time is thereafter automatically registered as Processing as harvesting (activitiy in harvesting head and loader/linkage) is resumed when machine is restarted.

- A down time in harvesting, due to a telephone call during 8 minutes, is not registered as a Down time since it is shorter than 15 minutes (Down-filter-time), this time is consequently included in Processing and therefore in the Main work time (E15). These 8 minutes are, however, not included in the E0-time since it is longer than 15 seconds (Minimum-filter-time).

- The harvester is to be moved to a new harvesting site before the work day is over. The machine is driven there by itself for 35 minutes, no trailer transport is needed. The operator therefore manually register this time as Road travel (Bm).

- Time registration is finished at 16.45 when the operator logs out of the computer and shuts down the machine. This means that the time therafter is considered to be Lost time (U).

# Registration of working-hours

The issue of handling registration of working-hours used for calculating wages was discussed during the development of this standardisation. Both machine manufacturers and contractors showed an interest in computer applications where work time registration and operational monitoring is integrated. A structure OperatorWork time is therefore included in the standard. The time registered in this variable shall hold the total working-hours including time spent separate from / outside the machine. An employee that do not work on the machine at all should be able to register his working-hours in the element OperatorWorkTimes.

Figure 9c. Schema structure for registering operator work times. Observe that there is a choice between OperatorIndividualWorkTime and IndividualMachineWorkTimeKey whereby the operator can register work outside the machine by using the OperatorIndividualWorkTime.

The following example describes the difference between IndividualMachineWorkTime and OperatorIndividualWorkTime if the work time of two different operators (driving the same harvester) are to be registered in one mom-file:

Operator 1: 				Operator 2:

IndividualMachineWorkTime  = 8.00 – 12.00	IndividualMachineWorkTime = 12.00 – 18.00

OperatorIndividualWorkTime = 8.00 – 14.00 	OperatorIndividualWorkTime = 11.00 – 18.00

Observe that the planning in this case involves work outside the harvester.

# OperatorShift

Traditionally many reports for analysing productivity figures are presented per shift or type of shift. The mom file therefore includes a structure for reporting when a specific shift was started and ended based on mom data Parallel with OperatorWorkTimes (figure 10). Observe that there is no strict definition of a shift, this means that it is probably implemented in different ways by different manufacturers. It is also an optional structure which means that it has not been implemented by all systems supporting the mom-format.

Figure 10. Schema structure for registering the start and end of an operator work shift.

# OperatorLogInEvent

The old OperatorLoginTime was complex to implement and interpret as the same StartTime, but with consequently increasing TimeLenght, is reported if several mom-files are created for a whole work day.

The OperatorLoginTime has been replaced with a simpler structure OperatorLoginEvent whereby the time is logged each time the operator logs in or out of the controller using the operatorLoginEventCategory (enumeration values are Login and Logout) as illustrated in figure 11.

Figure 11. Schema structure for registering when an operator logs in and out of the controller.

# Tracking

Modern mom-files include the possibility of enclosing tracking data which means that the movements of the machines can be reported. A common algorithm is to register one new position once the machine has moved a certain distance. It is strongly recommended that the time stamp in CoordinateDate is always included with the mom-files.

Figure 12. Schema structure for registering machine tracking data.

# Code list

All codes for e.g. down times were strictly defined as enumeration values in the mom-schema in earlier version of the StanForD standard (prior to 4.0).

The codes and free text names used when defining the reasons for other work, down times, unutilized times and operator work times are today included in an mcl-file. The mcl-file is based on the OperationalMonitoringCodeList-schema (figure 13). The default codes, which are identical with the previous standard version, are presented in appendix 2 (StanForD_Default_v1_English.mcl).

The reason for this new construction is that it makes operational monitoring much more flexible and agile. Depending in market needs the number of codes can be very easily adjusted without having to wait for a new standard version.

Figure 13. Schema structure for registering a code list in OperationalMonitoringCodeList-schema

Figure 14. Illustration of how a specific code list is identified in a mom-file.

# F.A.Q.

Question 1.

How are repairs that last for a long period of time, e.g. for four days between Monday and Thursday, handled?

Answer 1.

The normal planned time when machine is supposed to be carrying out work, in this case for four days, should be registered as Repair time, excluding hours when machine is normally standing, for example during the night. The reason for Repair time must of course also be defined according to the standard.
In this case it means that, if the machine usually is used for 14 hours per day, the registered Repair time will be 4 * 14 = 56 hours.

Question 2.

The operational monitoring is done per operator. Operator 1 finishes his shift with 8 minutes of Repair time (prior to this Processing) and operator 2 starts the next shift immediately afterwards with 9 minutes of Repair time before he resumes Processing.
Will these 17 minutes of repair time be registered as a Down time?

Answer 2.

No, the down time will not be registered if the operational monitoring is carried out per operator. This means that, in both cases, the time will be registered as Processing since we are monitoring each operator separately. The Repair times will be included in the Main work time (E15) but not in the E0-time.

The example is based on the assumption that the Main-filter-time is 15 minutes. Both Down times would have been registered if Main-filter-time had been set to 7 minutes.

# References

Anon. 1969 Time concepts for machine work. Skogsarbeten, Stockholm, Sweden·

Anon. 1978 Nordic agreement on forest work study nomenclature. Nordiska Skogsarbetssturiernas råd. Norsk Institutt for skogforskning, Ås, Norge.

Ämnesord:

# Appendix 1. StanForD variables

All operational monitoring elements and attributes except those included under SpeciesgroupDefinition and UserDefinedData are included in table below. Observe that this table has not been completely updated according to StanForD version 4.

| Name | In | Unit | Description |
| --- | --- | --- | --- |
| Air | CarrierRepairReason | list | Enumeration list with air repair reasons. Allowed values are: "Not defined","Compressor","Valves","Pipes","Anti-freeze (coolant) equipment" and "Miscellaneous" |
| ApplicationVersionCreated | MessageHeader | free text | Name and version of software application that created and saved a message |
| ApplicationVersionModified | MessageHeader | free text | Name and version of software application that modified and saved message |
| areaUnit | MessageAttributeGroup | list | Unit of area information in StanForD 2010 messages.Only allowed value: ha |
| AuditingOrganisation | ObjectDefinition |  | Contact information for organisation responsible for 3rd party control of measuring accuracy |
| baseModelYear | MachineBaseModel | Year | Manufacturing year of base machine |
| CarrierRepairReason | Repair |  | Structure with repair reasons for carrier / base machine |
| Code | ManufacturerCode |  | Manufacturer or user defined codes |
| CodeDescription | ManufacturerCode |  | Manufacturer or user defined code descriptions. |
| CombinedEndTime | CombinedMachineWorkTime | * | End time for the last of all combined times |
| CombinedMachineDownTime | CombinedMachineWorkTime |  | Structure with down times for machine. |
| CombinedMachineRunTime | CombinedMachineWorkTime |  | Structure with run times for machine. Several individual run times can me aggregated. |
| CombinedMachineWorkTime | MachineOperationalMonitoring |  | Structure with run, down and untilized times. |
| CombinedUnutilizedTime | CombinedMachineWorkTime |  | Structure with unutilized times for machine. |
| ContactInformation | LoggingOrganisation |  | Name and other contact information of logging organisation |
| ContactInformation | OperatorDefinition |  | Operator name and other possible contact information |
| ContractCategory | ContractNumber | free text | Category of contract |
| ContractNumber | ObjectDefinition | free text | Harvesting object contract number/identity |
| CountryCode | MessageHeader | code | Country code for computer generating message Country code: numerical code as per Swedish Std. SS-ISO 3166. Eg. Australia=036, Finland=246, France=250, Germany=276, Sweden=752 |
| CreationCoordinates | MessageHeader |  | Coordinates of machine when creating file, ie position at the time register in CreationDate. |
| CreationDate | MessageHeader | * | Date when message was created and saved |
| DeliveryDefinitionEstimation | ForwarderData |  | Defines primary unit of forwarded products |
| DeliveryEstimationMethod | DeliveryEstimation | list | Enumeration value describing how forwarder production data was generated per dlivery definition. Allowed values are: "Manual estimation by operator", "Scale", "Estimation through Harvested Production message" and "Other" |
| DeliveryEstimationUnit | DeliveryEstimation | list | Enumeration value describing the unit wich was estimated by operator or measured by forwarder (not calculated by machine) per delivery definition. Allowed values are: "Volume, m3sob", "Volume, m3sub", "Green mass, kg","Number of logs", "Number of loads", "Loose volume, m3", "Solid volume including bark, branches and needles, m3", "Solid volume of bundles (length*cross sectional area), m3" and "Number of bundles" |
| diameterUnit | MessageAttributeGroup | list | Unit of diameter information in StanForD 2010 messages.Only allowed value: mm |
| distanceClass | ExtrapolationCoefficient | cm | Distance class values (distance from the felling cut) in extrapolation coefficient table per species group. First value is 0 cm and the last is the value of the actual reference height in element ReferenceHeight (e.g. 130 cm). Intervals normally 10 cm. |
| District | LoggingOrganisation | free text | Name of district |
| Disturbance | MachineDownTime |  | Structure with reasons for disturbances |
| DisturbanceManufacturerCode | Disturbance |  | Manufacturer or user defined codes for disturbances. |
| DisturbanceStandardCode | Disturbance | list | Enumeration list with reason for disturbances. Allowed values are "Miscellaneous / other","Planning/follow up","Machine stuck","Weather","Ordered stop" |
| DrivenDistance | OtherMachineData | m | Covered distance during time included in MachineWorkTime. |
| Electrical | CarrierRepairReason | list | Enumeration list with electrical repair reasons. Allowed values are: "Not defined","Alternator","Starter","Harnesses","Electrical connection box","Instruments","Working lights","Controls","Sensor(s)","Solenoide","Chair equipment","Communication equipment","Fire extinguisher","Miscellaneous" |
| Electrical | HarvestingHeadRepairReason | list | Enumeration list with electrical repair reasons. Allowed values are: "Not defined","Harnesses","Controls","Bucking computer","Sensor","Solenoide","Miscellaneous" |
| Electrical | LoaderLinkageRepairReason | list | Enumeration list with electrical repair reasons. Allowed values are: "Not defined","Harnesses","Controls","Working lights","Sensor(s)","Solenoide","Miscellaneous" |
| EndDate | ObjectDefinitionMachine | * | Time and date for ending a site. Mandatory when object is finished |
| EngineTime | OtherMachineData | seconds | Engine time during time included in MachineWorkTime. |
| EstonianVolumeParameters | SpeciesGroupDefinitionMachine |  | Structure for parameters used in Estonian function for volume calculation. This function is also called Nilson's formula. See appendix |
| ExtrapolationCoefficient | ButtEndProfileExtrapolationTable |  | Extrapolation coefficient tables: coefficient values (multiplying values) / distance class / diameter class / tree species: 1â€¦var170_t4 / 1 â€¦ var170_t3 / 1â€¦var111_t1. Example: Measured diameter at reference height (normally dbh) of stem = 23,3 cm. Distance to extrapolation point = 90 cm. Coefficient value in table at (23, 90) = 1.042. Estimated diameter value at 90 cm = 23,3 cm * 1.042 = 24,3 cm (243 mm). |
| FactorB | SoundKnotFunction |  | Factor for determining the limit for sound knots/tree species (abbreviation "b" in y = (a+b*x+c*X2)*d), where diameter of the sound knot cylinder = DBH*y |
| FactorB | SwedishZacco | positive decimal | Factor b in the following linear function: Double bark thickness = a + b * diameter ob Bark thicknes and diameter in mm. |
| FactorC | SoundKnotFunction |  | Factor for determining the limit for sound knots/tree species (abbreviation "c" in y = (a+b*x+c*X2)*d), where diameter of the sound knot cylinder = DBH*y |
| ForestCertification | ObjectDefinition | list | Harvesting object certified. Forest certifications as enumeration. Allowed values: "FSC", "PEFC", "Other", "None", "Not known" |
| ForestOwner | ObjectDefinition |  | Seller of the right to harvest a forest/object. Person or organisation owning forest that is harvested by logging organisation. Observe that ForestOwner and LoggingOrgansiation may be identical. |
| ForwardedGreenMass | ForwarderData | kg | Total forwarded mass |
| forwardedVolumeCategory | TotalForwardedVolume | list | Defines volume registered in TotalForwardedVolume. Allowed values: "Volume, m3sob", "Volume, m3sub", "Loose volume, m3", "Solid volume including bark, branches and needles, m3", "Solid volume of bundles (length*cross sectional area), m3" |
| ForwarderData | OtherMachineData |  | Forwarder production, fuel and distance |
| FrequencyShortDownTimes | MachineOperationalMonitoring |  | Structure for for short down times register as an aggregated frequency table. |
| FuelConsumption | OtherMachineData | litre | Fuel consumption during time included in machine work time. |
| German | BarkFunction |  | Structure for Swedish Zacco bark function. |
| harvestedLogsVolumeCategory | TotalVolumeOfHarvestedLogs | list | Allowed values: "m3sob", "m3sub", "m3sobEstimated", "m3subEstimated" |
| HarvesterData | OtherMachineData |  | Structure for harvester production data in mom |
| HarvestingHeadRepairReason | Repair |  | Repair resons for harvesting head. |
| headModelYear | MachineHeadModel | Year | Manufacturing year of harvester head. |
| Hydraulics | CarrierRepairReason | list | Enumeration list with hydraulic repair reasons. Allowed values are: "Not defined","Pump","Hydraulic motor","Hose (pipe)","Valves","Hydraulic cylinders","Controls","Filter","Miscellaneous" |
| Hydraulics | HarvestingHeadRepairReason | list | Enumeration list with hydraulic repair reasons. Allowed values are: "Not defined","Pipes (hoses)","Valves","Hydraulic cylinder","Saw motor","Feed roller motor","Chain tensioner","Miscellaneous" |
| Hydraulics | LoaderLinkageRepairReason | list | Enumeration list with hydraulic repair reasons. Allowed values are: "Not defined","Hose (pipe)","Valves","Hydraulic cylinder","Motor","Rotator","Miscellaneous" |
| IndividualMachineDownTime | IndividualMachineWorkTime |  | Structure for down times. |
| IndividualMachineRunTimeCategory | IndividualMachineWorkTime | list | Allowed values: "Processing", "Terrain travel", "Other work" |
| IndividualMachineWorkTime | MachineOperationalMonitoring |  | Structure for machine work times including run, down and unutilized times. |
| IndividualShortDownTime | MachineOperationalMonitoring |  | Structure for individual short down times. Not aggregated. |
| IndividualUnutilizedTimeCategory | IndividualMachineWorkTime | list | Allowed values: "Meal break", "Other unutilized times" |
| Latitude | Skogforsk2004ScotsPine | latitude | Latitude used in bark function Should be set in machine since it is specific for certain geographical area.. |
| latitudeCategory | Latitude | list | Allowed values: "North", "South". Registered according to coordinateReferenceSystem and receiverPosition. |
| lengthUnit | MessageAttributeGroup | list | Unit of length information in StanForD 2010 messages.Only allowed value: cm |
| LoaderLinkageRepairReason | Repair |  | Structure with repair reasons for loader or linkage. |
| loadNumberOfCategory | LoadNumberOfItems | list | Enumeration values defining the actual content of the element. Allowed values are: "Estimated no of logs" and "Number of bundles" |
| LoadNumberOfItems | ForwarderData | no | Number of loads or bundles |
| LoggingContractor | Machine |  | Contact information of machine contractor |
| LoggingForm | ObjectDefinition |  | Defines possible loggin form of object |
| LoggingForm | SubObjectDefinition |  | Defines possible loggin form of subobject |
| LoggingFormCode | LoggingForm | code | Code for logging form |
| LoggingFormDescription | LoggingForm | free text | Description of logging method |
| LoggingOrganisation | ObjectDefinition |  | Contact information for organisation responsible for logging an object (normaly forest company or forest owners association), owner of the right to harvest a certain object. Old types Organisation, Region, District, Working team, Wood pile |
| longitudeCategory | Longitude | list | Allowed values: "East", "West". Registered according to coordinateReferenceSystem and receiverPosition. |
| lowerDiameterLimit | BarkDeduction | mm | Lower diameter limits per species group: Bark function based on diameterclasses with fixed bark deductions (double), based on German requirements |
| LowerTimeLimit | FrequencyShortDownTimes | seconds | Lower time limit for registering short down times. Interval normally 60 seconds |
| Machine | OperationalMonitoring |  | All the oprational monitoring specific data is included under Machine elements |
| MachineApplicationVersion | Machine | free text | Name and version of main software application in forest machine |
| MachineBaseManufacturer | Machine | free text | Machine manufacturer. Free text that describes the machine manufacturer. |
| MachineBaseModel | Machine | free text | Machine model. Free text that describes the model of the machine. |
| machineCategory | Machine | list | Enumeration list describing type of machine: Harvester, Forwarder, Harwarder, Bundler, Scarifier, Other |
| MachineDrivenDistance | MachineOperationalMonitoring | m | Total distance covering the total life time of the machine up until the time of the report end time. |
| MachineEngineTime | MachineOperationalMonitoring | minutes | Total engine time, covering total life time of the engine up until the time of the report end time. |
| MachineFuelConsumption | MachineOperationalMonitoring | litre | Total fuel consumption covering the total life time of the machine up until the time of the report end time. |
| MachineHeadManufacturer | Machine | free text | Harvester head manufacturer. Free text that describes the head manufacturer. |
| MachineHeadModel | Machine | free text | Harvester head model. Free text that describes the model of the harvester head. |
| MachineKey | Machine | free text | Machine specific globally uniqe identity (GUID). Must be updated if memory of previously used Keys are lost. Possible for manufacturers to use this in order to identify indivudal machines. Other users of data should use MachineUserId or MachineIdOwner. |
| MachineOwner | Machine |  | Contact information of machine owner |
| MachineOwnerID | Machine | free text | Machine identity set by machine owner. Identification that may be used by machine owner. |
| MachineRunTimeCategory | CombinedMachineRunTime | list | Allowed values: "Processing", "Terrain travel", "Other work"" |
| MachineUserID | Machine | free text | Machine identity set by user of data. Normally to be used and decided by logging organisation in order to identify a certain machine. For example necessary when paying forest on based on machine measurement. Registered in machine. |
| Maintenance | MachineDownTime |  | Structure with maintenance reasons |
| MaintenanceManufacturerCode | Maintenance |  | Manufacturer or user defined maintenance reasons |
| MaintenanceStandardCode | Maintenance | list | Enumeration list with maintenance reasons. Allowed values are: "Other maintenance","Control and calibration","Saw maintenance","Refilling and lubrication" |
| Mechanical | CarrierRepairReason | list | Enumeration list with mechanical repair reasons. Allowed values are: "Not defined","Frame","Hitch, oscillation joint","Front axle","Rear axle","Bogies","Tracks and chains","Brake system","Diesel engine","Gearbox","Fuel system","Cab","Seat","Fire extinguisher","Miscellaneous" |
| Mechanical | HarvestingHeadRepairReason | list | Enumeration list with mechanical repair reasons. Allowed values are: "Not defined","Frame","Guarding","Feed roller arms","Feed roller","Delimbing knives","Saw unit","Chain tensioner","Chain lubrication","Measuring devices","Color marking","Stump treatment","Miscellaneous" |
| Mechanical | LoaderLinkageRepairReason | list | Enumeration list with mechanical repair reasons. Allowed values are: "Not defined","Pillar base / slewing device","Post","Boom","Stick","Telescopic extension","Grapple","Damping linkage","Miscellaneous" |
| messageType | OperationalMonitoring | list | Enumeration list of file extensions of StanForD 2010 messages: Allowed value in Operational Monitoring message is: "mom" |
| modificationAuthor | ModificationDate | free text | Author of modification |
| ModificationDate | MessageHeader | * | Date when message was last modified and saved |
| MonitoringFilterTimeDown | MonitoringSettings | seconds | Down-filter-time, maximum time allowed for a down time, if down time is longer it will be recorded in MachineDownTime. Default value is 900 s (15 min). The value rules the data in MonitoringFilterTimeMinimum |
| MonitoringFilterTimeMinimum | MonitoringSettings | seconds | Minimum-filter-time: Smallest considered time unit. If any time (run time or down time) is shorter it is considered non-existing. Default value is 15 s. This means for example that a down time shorter than this is included in the E0-time, if longer it will not be included in the E0-time |
| MonitoringFilterTimeRun | MonitoringSettings | seconds | Run-filter-time, A Main work time (Processing, Terrain travel, Other work or Road travel) in MachineWorkTime, must exceed this filter time in order to be registered as a new Main work time.. Default value is 120 s (2 min). This means e.g. that a run time shorter than this will be considered to be of the same type of time as the previously registered time. |
| MonitoringSettings | MachineOperationalMonitoring |  | Structure for monitoring filter times. |
| MonitoringSettingsKey | CombinedMachineWorkTime | sequential no | Machine specific identity per monitoring settings |
| MonitoringSettingsKey | IndividualMachineWorkTime | sequential no | Machine specific identity per monitoring settings |
| MonitoringSettingsKey | MonitoringSettings | sequential no | Machine specific identity per monitoring settings |
| MonitoringStartTime | IndividualShortDownTime | * | Start time for a time interval |
| MonitoringStartTime | MonitoringTime | * | Start time for a time interval |
| MonitoringStartTime | OperatorLoginTime | * | Start time for a time interval |
| MonitoringTimeLength | IndividualShortDownTime | seconds | Extension of a time interval |
| MonitoringTimeLength | MonitoringTime | seconds | Extension of a time interval |
| MonitoringTimeLength | OperatorLoginTime | seconds | Extension of a time interval |
| NoOfShortDownTimes | FrequencyShortDownTimes | no | Number of short down times per time interval |
| NumberOfHarvestedStems | HarvesterData | no | Number of processed (harvested) stems. |
| NumberOfLoads | ForwarderData | no | Number of loads unloaded |
| ObjectArea | ObjectDefinition | ha | Area of harvesting object |
| ObjectDefinition | MachineOperationalMonitoring |  | Structure for object specific data |
| ObjectKey | CombinedMachineWorkTime | sequential no | Machine specific identity per harvesting object. Set when new harvesting object is started. Running no recommended, reset only when MachineKey is changed |
| ObjectKey | MonitoringTime | sequential no | Machine specific identity per harvesting object. Set when new harvesting object is started. Running no recommended, reset only when MachineKey is changed |
| ObjectKey | ObjectDefinitionMachine | sequential no | Machine specific identity per harvesting object. Set when new harvesting object is started. Running no recommended, reset only when MachineKey is changed |
| ObjectKey | OperatorShiftDefinition | sequential no | Machine specific identity per harvesting object. Set when new harvesting object is started. Running no recommended, reset only when MachineKey is changed |
| ObjectKey | ShortDownTime | sequential no | Machine specific identity per harvesting object. Set when new harvesting object is started. Running no recommended, reset only when MachineKey is changed |
| ObjectName | ObjectDefinition | free text | Descriptive name of harvesting object |
| ObjectUserID | ObjectDefinition | free text | Identity per harvesting object defined by user of data, normally logging organisation. |
| OperationalMonitoring |  |  | The root element of the Operational Monitoring message |
| OperationalMonitoringHeader | OperationalMonitoring |  | General information for message |
| OperatorDefinition | Machine |  | Structure for operator data. |
| OperatorKey | CombinedMachineWorkTime | sequential no | Machine unique operator identity, set by machine |
| OperatorKey | MonitoringTime | sequential no | Machine unique operator identity, set by machine |
| OperatorKey | OperatorDefinition | sequential no | Machine unique operator identity, set by machine |
| OperatorKey | OperatorLoginTime | sequential no | Machine unique operator identity, set by machine |
| OperatorKey | OperatorShiftDefinition | sequential no | Machine unique operator identity, set by machine |
| OperatorKey | Row | sequential no | Machine unique operator identity, set by machine. Operatorkey should be updated when a row is modified. |
| OperatorKey | ShortDownTime | sequential no | Machine unique operator identity, set by machine |
| OperatorLogInCoordinates | OperatorLoginTime |  | coordinates of the machine at the time of log-in |
| OperatorLoginTime | MachineOperationalMonitoring |  | Structure for register when operator is logged into machine software system. |
| OperatorLogOutCoordinates | OperatorLoginTime |  | coordinates of the machine at the time of log-out |
| OperatorShiftDefinition | MachineOperationalMonitoring |  | Structure for operator shifts |
| OperatorUserID | OperatorDefinition | free text | Identity of operator defined by user of data, set in machine |
| OperatorWorkTime | MachineOperationalMonitoring |  | Structure for operator work times, including work outside machine. |
| OperatorWorkTimeCategory | OperatorWorkTime | list | Allowed values: "MachineRunTime", "Waiting for repair", "Trailer transportation", "Meal break", "Planning outside machine", "Travel to work outside machine", "Other work outside machine" |
| OtherMachineData | CombinedMachineWorkTime |  | Structure for production, fuel, distance and engine time. |
| OtherMachineData | IndividualMachineWorkTime |  | Structure for production, fuel, distance and engine time. |
| OtherMachineDownTimeCategory | MachineDownTime |  | Structure for other down times. |
| OtherMachineDownTimeManufacturerCode | OtherMachineDownTime |  | Manufacturer or user defined codes. |
| OtherMachineDownTimeStandardCode | OtherMachineDownTime | list | Allowed values: "Waiting for repair", Trailer transportation","Unproductive terrain work", "Other" |
| OtherRepairReason | Repair | list | Allowed values: "Other", "Not definded" |
| otherWorkCategory | MachineRunTimeCategory | list | Allowed values:"Road travel","Preparing strip roads","Towing other machine","Roadside loading of truck","Unspecified"" |
| ProcessingCategory | HarvesterData | list | Possible enumeration values are: SingleTreeProcessing,MultiTreeProcessing, SIngleTreeFelling and MultiTreeFelling |
| RealEstateIDObject | ObjectDefinition | free text | Real estate identity for object. |
| RealEstateIDSubObject | SubObjectDefinition | free text | Real estate identity for sub-object. |
| ReferenceHeight | ButtEndProfileExtrapolation | cm | Height from stump to the diameter used for predicting the butt end diameters. Normally breast height (130 cm in Finland, 120 cm in Sweden) is used for extrapolation of butt end diameter values. |
| RegistrationNumber | Machine | free text | Registration number of machine |
| Repair | MachineDownTime |  | Structure with repair reasons |
| RepairManufacturerCode | Repair |  | Manufacturer or user defined codes for repair reasons |
| ReportEndTime | ReportInterval | * | End date for monitoring report. Use ReportStartTime and ReportEndTime to calculate calender time. |
| ReportFilterCategory | ReportInterval | tlist | Possible enumeration values are: "No filter", "Object filter" and "Other filter". No filterâ€ means that â€œAll run (work) times, down times and meal breaks that occurred between ReportStartTime and ReportEndTime are included, with no exceptions. This means that any times not registered are to be considered to be unutilized.â€ The value â€œObject filterâ€ means that only times directly connected to a specific object are included. |
| ReportInterval | MachineOperationalMonitoring |  | Total time interval covered by the mom file. |
| ReportStartTime | ReportInterval | * | Start date for monitoring report. Use ReportStartTime and ReportEndTime to calculate calender time. |
| SenderApplication | MessageHeader | free text | Name and version of application sending the message |
| ShiftKey | OperatorShiftDefinition | sequential no | Machine unique shift identity, set by machine |
| ShiftKey | OperatorWorkTime | sequential no | Machine unique shift identity, set by machine |
| ShiftCategory | OperatorShiftDefinition | list | Shift type allowed values are: "Not defined", "Morning", "Day", "Evening" and "Night" |
| ShiftDescription | OperatorShiftDefinition | free text | Description of shift |
| ShiftEndTime | OperatorShiftDefinition | * | End time of the shift |
| ShiftOrder | OperatorShiftDefinition | sequential no | Order number of the shift (First order is 1, seciond is 2 and so on) |
| ShiftStartTime | OperatorShiftDefinition | * | Start time of the shift |
| SparePartDescription | SpareParts | free text | Descriptive information of the spare part |
| SparePartIdentity | SpareParts | free text | Identity information of the spare part |
| SpareParts | MachineDownTime |  | Structure for spare parts |
| SparePartsNoOfItems | SpareParts |  | Number of spare parts |
| SpeciesGroupKey | HarvesterData | sequential no | Machine specific identity per species group set by machine. Must be updated if any modifications to species group definition is done. Running number reset only when MachineKey is changed. |
| StartDate | ObjectDefinitionMachine | * | Time and date for starting production at a site for the first time. |
| StartGrade | Grades |  | Default grade per species group when starting to process the stem in the harvester. |
| StartTime | CombinedMachineDownTime | * | Start time for a number of combined times or for individual times |
| StartTime | CombinedMachineWorkTime | * | Start time for a number of combined times or for individual times |
| StemTypeCode | StemDefinition | free text (usually finish PMP-codes) | Code for stem type |
| StemTypeName | StemDefinition | free text | Name/Description of stem type |
| SubObject | ObjectDefinitionMachine |  | Subobject specific information |
| SubObjectArea | SubObjectDefinition | ha | Area of harvesting subobject |
| SubObjectKey | CombinedMachineWorkTime | sequential no | Machine specific identity per harvesting sub object, set by machine. Set when new harvesting sub object is started. Running no recommended, reset when ObjectKey is updated. |
| SubObjectKey | MonitoringTime | sequential no | Machine specific identity per harvesting sub object, set by machine. Set when new harvesting sub object is started. Running no recommended, reset when ObjectKey is updated. |
| SubObjectKey | OperatorShiftDefinition | sequential no | Machine specific identity per harvesting sub object, set by machine. Set when new harvesting sub object is started. Running no recommended, reset when ObjectKey is updated. |
| SubObjectKey | ShortDownTime | sequential no | Machine specific identity per harvesting sub object, set by machine. Set when new harvesting sub object is started. Running no recommended, reset when ObjectKey is updated. |
| SubObjectKey | SubObjectDefinitionMachine | sequential no | Machine specific identity per harvesting sub object, set by machine. Set when new harvesting sub object is started. Running no recommended, reset when ObjectKey is updated. |
| SubObjectName | SubObjectDefinition | free text | Descriptive name of sub harvesting object |
| SubObjectUserID | SubObjectDefinition | free text | Identity per sub harvesting object defined by user of data, normally by logging organisation. |
| SwedishZacco | BarkFunction |  | Structure for Swedish Zacco bark function. |
| Team | LoggingOrganisation | free text | Name of team |
| TextFromMachine | ObjectDefinitionMachine | free text | Text related to harvesting object sent from machine |
| TimeLength | CombinedMachineDownTime | seconds | Extension of a time element |
| TimeLength | CombinedMachineRunTime | seconds | Extension of a time element |
| TimeLength | CombinedUnutilizedTime |  | Extension of a time element |
| TotalForwardedVolume | ForwarderData |  | Total forwarded volume. |
| TotalVolumeOfHarvestedLogs | HarvesterData |  | Total solid volume (including unclassified volumes) |
| UnutilizedTimeCategory | CombinedUnutilizedTime |  | Allowed values: "Meal break", "Other unutilized times" |
| UpperTimeLimit | FrequencyShortDownTimes | seconds | Lower time limit for registering short down times. Interval normally 60 seconds |
| UserDefinedData | MachineOperationalMonitoring |  | Structure for user defined data |
| weightUnit | MessageAttributeGroup | list | Enumeration list of file extensions of StanForD 2010 messages.Only allowed value: kg |
| version | MessageAttributeGroup | version number | The version number of the message in format: "major"."minor", where "major" is a sequential number of the major version (start from 1) and "minor" is a sequential number of the minor version.> |
| versionDate | MessageAttributeGroup | yyyy-mm-dd | Date of version |
| volumeUnit | MessageAttributeGroup | list | Unit of volume information in StanForD 2010 messages. Only allowed value: m3 |

* yyyy-mm-ddThh:mm:ss[+|-]hh:mm

# Appendix 2. Enumeration lists for repair causes

The codes below are identical with the previous standard version. From version 4 enclosed in StanForD_Default_v1_English.mcl.

| CarrierRepairReason |  |  | LoaderLinkageRepaireReason |  | HarvestingHeadRepaireReason |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | Part |  | Type | Part | Type | Part |
| Electrical | Alternator |  | Electrical | Harnesses | Electrical | Harnesses |
|  | Starter |  |  | Controls |  | Controls |
|  | Battery (missing in mom) |  |  | Working lights |  | Bucking computer |
|  | Harnesses |  |  | Sensor(s) |  | Sensor |
|  | Electrical connection box |  |  | Solenoide |  | Solenoide |
|  | Instruments |  |  | Miscellaneous |  | Miscellaneous |
|  | Working lights |  | Hydr. | Hose (pipe | Hydr. | Pipes (hoses) |
|  | Controls. |  |  | Valves |  | Valves |
|  | Sensor(s) |  |  | Hydraulic cylinder |  | Hydraulic cylinder |
|  | Solenoide |  |  | Motor |  | Saw motor |
|  | Chair equipment |  |  | Rotator |  | Feed roller motor |
|  | Communication equipment |  |  | Miscellaneous |  | Chain tensioner |
|  | Fire extinguisher |  | Mech. | Pillar base / slewing device |  | Miscellaneous |
|  | Miscellaneous |  |  | Post | Mech. | Frame |
| Hydr. | Pump |  |  | Boom |  | Guarding |
|  | Hydrualic motor |  |  | Stick |  | Feed roller arms |
|  | Hose (pipe) |  |  | Telescopic extension |  | Feed roller |
|  | Valves |  |  | Grapple |  | Delimbing knives |
|  | Hydraulic cylinders |  |  | Damping linkage |  | Saw unit |
|  | Controls. |  |  | Miscellaneous |  | Chain tensioner |
|  | Filter |  |  |  |  | Chain lubrication |
|  | Miscellaneous |  |  |  |  | Measuring devices |
| Mech. | Frame |  |  |  |  | Color marking |
|  | Hitch, oscillation joint |  |  |  |  | Stump treatment |
|  | Front axle |  |  |  |  | Miscellaneous |
|  | Rear axle |  |  |  |  |  |
|  | Bogies |  |  |  |  |  |
|  | Tracks & chains |  |  |  |  |  |
|  | Brake system |  |  |  |  |  |
|  | Diesel engine |  |  |  |  |  |
|  | Gearbox |  |  |  |  |  |
|  | Fuel system |  |  |  |  |  |
|  | Cab |  |  |  |  |  |
|  | Seat |  |  |  |  |  |
|  | Fire extinguisher |  |  |  |  |  |
|  | Miscellaneous |  |  |  |  |  |
| Air | Compressor |  |  |  |  |  |
|  | Valves |  |  |  |  |  |
|  | Pipes |  |  |  |  |  |
|  | Anti-freeze (coolant) equip. |  |  |  |  |  |
|  | Miscellaneous |  |  |  |  |  |

# Appendix 3. Enumeration lists for disturbances and maintence.

The codes below are identical with the previous standard version. From version 4 enclosed in StanForD_Default_v1_English.mcl.

Enumeration lists for disturbances.

| Name | Description |
| --- | --- |
| Miscellaneous / other | Distrubances not defined |
| Administration, telephone | For example handling of apt-files, contacts with employer, phone calls, production reporting |
| Planning/follow up | For example pre-harvest planning of site, post harvest assessment of site, reporting of assessment |
| Machine stuck | Machine is stuck (standstill), for example due to wet and soft ground conditions |
| Weather | Extreme weather, for example low temperature, resulting in stand stills |
| Ordered stop | For example visitors, limitation of production. |

Enumeration lists for maintenance.

| Name | Description |
| --- | --- |
| Other maintenance | Maintenance not defined |
| Control and calibration | Control measurement and length/diameter calibration |
| Saw maintenance | Changing saw chain or saw bar |
| Refilling and lubrication | Lubrication of machine or refilling of fuel/fungicides/colors etc |
| Preventive maintenance | Daily check list, weekly inspection, hoses position, check hydraulic pressure etc. Could in the future be based on notifications from machine condition monitoring system. |
| Periodic maintenance | For example maintenance every 250, 500, 1000, 1500, 2000 and 4000 hours, according to uptime maintenance planner. |
| Machine wash |  |
| Out of fuel | Waiting for refueling. For example used if refueling is dependent on separate service truck. |

Observe that special maintenance vehicles are used for carrying out maintenance on a large number of forest machines on certain markets (for example plantations of eucalypt).

Enumeration lists for other down times.

| Name | Description |
| --- | --- |
| Waiting for repair | Not defined |
| Trailer transportation | Machine is moved with trailer during which the operator is present. |
| Unproductive terrain work | Excessive time spent traveling from pile to pile, cribbing trail, pulling stumps, etc. (Introduced based on North American needs in 2011) |
| Other | Othe unspecified down times. |

# Appendix 4. Swedish interface.

The following translations to Swedish should be used in all monitoring systems developed for the Swedish market.

| English | Swedish |
| --- | --- |
| Calendar time (Ca) | Kalendertid |
| Disturbance (Id) | Störning |
| Down time | Avbrottstid |
| E(t) | G(t) |
| E0 | G0 |
| E15 | G15 |
| Effective work time (E) | Grundtid |
| Loading & unloading | Av- och pålastning (skotare) |
| Lost time (U) | Outnyttjad tid |
| Lunch/meal break (Um) | Rast |
| Main work time (M) | Egentlig arbetstid |
| Maintenance (Im) | Skötsel |
| Meal break | Rast |
| Other work (Bo) | Övrigt arbete |
| Other untulized time | Övrit outnyttjad tid |
| Percent down time | Avbrottsgrad |
| Percent effective utilisation | Driftgrad |
| Percent machine availability | Maskinell tillgänglighet |
| Percent machine availability | Utnyttjandegrad |
| Percent operational availability | Operativ tillgänglighet |
| Percent repair time | Reparationsgrad |
| Percent technical availability | Teknisk utnyttjandegrad |
| Processing (Mp) | Upparbetning |
| Repair (Irep) | Reparation |
| Road travel (Bm) | Flyttning mellan objekt |
| Terrain travel (Md) | Terräng-körning |
| Total repair time | Total reparationstid |
| Trailer-transport (Itr) | Trailertransport |
| Utilised time (To) | Utnyttjad tid |
| Waiting for repair time (Irw) | Väntan reparation |

# Appendix 5. Illustrations of machine parts.

# Appendix 6. Minimum filter time and machine indications

Introduction

In order to avoid operators being forced to define the reason for Down times in the case of short down times some kind of filtering is needed. The following filtering should be used by the monitoring system:

- UDown-filter-time (t)U: A Down time must exceed this filter time in order to be registered as a Down time. The suggested default value is 15 minutes (MonitoringFilterTimeDown).

- UMain-filter-timeU: A Main work time (Processing, Terrain travel, Other work(unspecified), Road travel, Preparing strip road, Towing other machin or Roadside loading truck) must exceed this filter time in order to be registered as a new Main work time. The suggested default value is 2 minutes (MonitoringFilterTimeRun). 
This means that if a Main work time is shorter than 2 minutes this time will be register as the previously register type of time.

- UMinimum-filter-timeU: A time (Main work time or Down time) being shorter this time is not to be consider at all. This filter time can be viewed as the smallest time unit used in the monitoring system. The suggested default value is 15 seconds (MonitoringFilterTimeMinimum).
This means that, for example, a Down time shorter than 15 seconds will be included in the E0-time while a Down time longer than 15 seconds will not be included in the E0-time.

If we, for example, receive one indication from the feed rollers within a ”Minimum filter time” (MFT), by default 15 seconds, we assume by definition that the feed rollers are active/moving during the whole time interval (MFT).

This means that, if we have any indications from both feed rollers and swing/crane during 15 seconds (MFT) all of these 15 seconds are considered to be processing time.

Examples

It is assumed that the default values (minimum filter time 15 seconds) are used in the following examples. However, observe that it must be possible to change the filtering times in the machine.

Example 1

Example 2

Example 3

Example 4

Example 5

Example 6

These 75 second will be recorded as Processing if this interval was preceded by more than two minutes of Processing.

If these indications had occurred right after start-up the preceding time would probably have been recorded as Unutilized time, meaning that these 75 seconds would have been recorded as Unutilized time.

Example 7

The first 60 seconds should be recorded as Unutilised time if the time prior to start-up was considered to be Unutilised time.

Example 8

The first 60 seconds should be recorded as Maintenance as the time prior to start-up was recorded as Maintenance.

* Operator specified (in monitoring system) that maintenance was carried out before start-up.

Example 9

Example 10

Only down times longer than 15 minutes are registered as down times. In this case Maintenance time will be registered as Processing since that was the previous valid time registered.

However, these 14 minutes shall not be included in the E0 time. Meaning that E0 is 14 minutes shorter than E15 in this case.

Example 11

* The user is asked (2 minutes after a new Main work time is started) for the reason for a Down time which was longer than 15 minutes. In this case the operator defined the time as Maintenance.

Example 12

These are the basic rules that we should always follow:
E(t) = Processing + Terrain travel + Other Work + Road travel + Preparing strip road + Towing other machine + Roadside loading truck
E(t) = E0 + short down times

A short down time coming directly after a longer down time (for example defined as repair) should be considered to be part of the longer down time (repair in this case). A short down time can only "exist" when the previous time is defined as work time.

Two different short down times are included in the diagram above. The first down time directly after a long down time (repair) will be part of this long down time since no long enough (>2 min) work is carried out. The second short down time will however be registered as a separate down time since it is preceded by some main work time.

Example 13

Frequent changes between processing and terrain travel does not change the time classification when shorter than the UMain-filter-time.

Example 14

Frequent changes between processing and terrain travel does not change the time classification when shorter than the UMain-filter-time. Terrain travel is not registered until it has lasted longer than the Main-filter-time.

Comments

We may very well have to increase the Minimum filter time (e.g. 15→20 sec) and decrease Main filter time (e.g. 120→90 sec) and Down filter time (e.g. 15 min→5 min). The present default values were originally based on experiences from SCA when they were using vibration recorders.

# Appendix 7.
Swedish instruction for time registration in operational monitoring of forest machines

Observe that this instruction, including the examples, was written with a Swedish perspective. It was written with the sole objective of being suitable under Swedish situations. It is not an official StanForD-document to necessarily be used by everyone using the monitoring system. However, the document is most certainly possible to use in most situations where operational monitoring systems are used. An example that may very well be specific for Sweden is how union activities are to be handled since all employees are, by law, allowed to take part in union activities during paid work time.

## Basic principles

### Introduction

Focus should primarily be on the machine and how the work team is performing at the work site. The objective with monitoring is to follow trends, to make comparisons and to make analyses based on facts. Monitoring should support development and improvements of:

- Machine systems

- Machine types and models

- The work of the team

- Organisations and enterprises

- Conditions at the work site

- Material for making prognoses

All time concepts in the standardized monitoring system (first implemented in 2005) have been structured according to figure 1. There are also definitions of a number of key figures included in order to facilitate comparisons of different machines.

Figure 1. Bar chart describing the structure of the time concepts.

### Registration of down times

Down times must be registered when operator is normally scheduled or available for driving the machine. This time should be considered as available machine time (this is equal to Utilised time as defined in document "Operational monitoring of forest machines under working condition").

Down times must be registered during available machine time if the down time is caused by (see also figure 2):

1. Activity carried out at work site

2. Insufficient planning of site

3. Machine (construction) or handling of machine

4. Activity carried out outside work site, if it is related to the machine or the handling of the machine

5. Climate, for example low temperature or thawing of frozen soil

One of the causes mentioned above is enough.

Figure 2. Description of when a down time should be registered.

Down times must not be registered during available machine time if caused by:

- Sickness

- Individual driver's decisions, like operating the machine at a different time

- Training or meeting away from work site if not related to machine or its handling (paragraph 4 above)

- Other activities not covered by paragraph 1-5 above.

Down time must have a priority order according to the following if several down times occurs "simultaneously":

- Repair

- Waiting for repair

- Maintenance

- Trailer transportation (during available machine time)

- Disturbance

- Meal break (considered to be unutilized time)

This order of priority means that for example a down time is to be registered as Waiting for repair even if maintenance is carried out while waiting for a spare part.

Down times that have several different causes in a sequence should be divided up into these causes when registered.

## Examples

A list of different possible cases has been compiled in order to demonstrate how the operator is to handle the monitoring system. The examples below are all assuming that the code list in StanForD_Default_v1_English.mcl is in use.

Example of cases and how they should be registered

| Case | Registration |
| --- | --- |
| Normal start of machine | Operator starts the computer and logs in, monitoring system starts automatically. |
| Shutting down machine | Operator automatically logged out when operator shuts down computer. Down time may be registered at next start up if operator did, for example, maintenance after shutting down. |
| Exchange of drivers | First operator logs out of machine computer and second operator logs in. |
| Main work time |  |
| Machine felling, delimbing and cutting stems | Automatically registered as Processing/loading |
| Machine loading and unloading logs | Automatically registered as Processing/loading |
| Machine drives to logging site or between login site and landing, including transportation of logs | Automatically registered as Terrain travel |
| Operator drives machine during weekend instead of normal work day | It should not matter when operator decides to work, the normal work day will thus not be registered (Unutilized time) while the weekend is considered to be a "normal" work day. |
| A stand-in operates machine | Time registered as usual, the stand-in may be registered as a new operator or as one of the regular operators |
| Other work |  |
| Laying out branches on strip road in order to improve bearing capacity | Registered as Other work |
| Salvaging or towing another machine | Registered as Other work |
| Helping another machine | Registered as Other work |
| Road travel |  |
| Machine driven (by itself) between two different work sites. | Driver manually defines (registers) the start and end time of Road travel, meaning no automatic registration of Road travel is done. Time will be registered as Road travel even if a tree is processed or the machine is coming to a complete stop as long as the operator does not manually end registration as Road travel. |
| Mounting or dismount tracks/chains when mowing machine. | All activities related to moving the machine between to site are included in Road travel for example dismounting tracks. |
| Machine moving by itself between two different work sites. On which work site is the time registered, is it divided up between the objects? | Time should normally belong to the site which the machine is leaving from as a new work site/object normally is created when harvesting/forwarding is started. Do not divide this time between different sites. It is however possible to define a new site prior to the arrival at the site, meaning that Road travel then may belong to the site to which the machine will arrive. |
| Maintenance |  |
| Regular maintenance according to manufacturer's instruction / schedule. | Registered as Maintenance |
| Maintenance carried out before machine/computer is started by operator | The operator should, after having started the computer and the monitoring system, register this time as Maintenance. |
| Same operator first repair machine and then carries out some maintenance | Registered as Repair and Maintenance, time should be divided up between these two. |
| Machine is started in advance (before the actual work is started) in order to pre-heat the engine (done under cold conditions) | Registered as Maintenance |
| Operator arrives before his scheduled work time and starts the machine in order to pre-heat the engine | If pre-heating is needed it should be registered as Maintenance irrespectively whether done before normal work time or not as the operator is available for driving at the site. |
| Machine is shut down and operator is refuelling and lubricating. | Registered as Maintenance, time registration done by restarting the computer before operator leaves site or when machine is started next day. |
| Trailer transportation |  |
| Machine is moved with trailer during which the operator is present. | Registered as Trailer transportation |
| Machine is moved with trailer during daylight, operators are taking part in training but are normally scheduled to drive machine. | Registered as Trailer transportation |
| Machine is moved with trailer outside normally scheduled work time for operators, operators are not present. | Time not registered |
| Repair & Waiting for repair |  |
| Machine repaired at workshop | Registered as Repair |
| Waiting for spare parts or service man. | Registered as Waiting for repair |
| Machine breaks down. | Time registered as Repair from the moment that the machine breaks down. However time should be registered as Waiting for repair if there is a delay due to waiting for spare part or repair man. |
| Machine started after having been repaired. | Time will automatically be registered as Processing or Terrain travel when machine is restarted. The driver should define the reason for the down time as Repair and its causes |
| Operator drives machine during weekend instead of normal work day | It should not matter when operator decides to work, the normal work day will thus not be registered (Unutilized time) while the weekend is considered to be a "normal" work day. |
| Operator waiting for service man who in his turn is waiting for spare part. | All down times which are caused by waiting for a spare part or a repair man are to be registered as Waiting for repair. |
| One operator repairs machine and a second operator carries out maintenance at the same time. | Registered as Repair. |
| One single operator first repairs machine and thereafter carries out maintenance. | Registered as Repair and Maintenance, time should be divided up between these two. |
| Machine repaired by operator during weekend, that is outside normal scheduled work time. | Registered as Repair. |
| Machine at repair shop, at the same time operators are at training or driving different machine. | Time should be register as Repair as it has the highest priority independently of what other activities the operators are carrying out. |
| Machine at repair shop during two normal work days. Normal scheduled work time in the forest is for example 2 days * 3 shifts * 8 hours while the actual work time at the shop is 2 days * 8 hours. | The actual repair time (2 days * 8 hours) in work shop is registered as Repair while the other tim as Waiting for repair (2days * 2 shifts * 8 hours). |
| Machine is repaired at repair shop during the weekend. | The actual repair time at the work shop is registered as Repair. Observe that this is an exception from the main rule that the operator should be available for driving the machine. |
| Disturbance |  |
| Changing of price list (apt-file) | Registered as Disturbance – Administration |
| Reporting of production (prd-file) | Registered as Disturbance – Administration |
| Telephone conversation | Registered as Disturbance - Administration |
| Training/meeting directly connected/related to the machine and the handling of the machine. | Registered as Disturbance - Stop |
| Salvaging of the machine | Registered as Disturbance – Stuck |
| Low temperatures makes it impossible to carry out work | Registered as Disturbance – Weather |
| High rainfall/precipitation makes it impossible to carry out work | Registered as Disturbance – Weather |
| Thawing of frozen soils makes it impossible to carry out work | Registered as Disturbance – Weather |
| Machine is standing as operator has to mark strip roads | Registered as Disturbance – Planning |
| Machine is standing as operator is doing a follow-up of a finished site. | Registered as Disturbance – Planning |
| Machine standing due to shortage of timber | Registered as Disturbance – Other |
| Machine standing due to obstacles at the landing | Registered as Disturbance – Other |
| Machine is standing as operator is doing pre-commercial cleaning of stand to be harvested | Registered as Disturbance – Other |
| Unutilized time |  |
| Machine is standing as operator is taking part in union activities | Time not registered |
| Machine is standing as operator is taking part | Time not registered |
| Vacation | Time not registered |
| Machine is standing as operator is sick | Time not registered |
| Machine is moved with trailer outside normally scheduled work time for operators, means that no operator is present. | Time not registered |
| One operator gets sick during the work day and machine is standing during part of the day | Time not registered |
| Operator going through training which is not related to machine or how it is handled. | Time not registered |